VOLLEDIGE CONFIGURATIES — v4

Copy-paste configs voor alle toestellen

Salaheddine Sennouni | Odisee 2026 | Groep 10 | Rack 1 | v4 — 18 juni 2026

========================================================================

WIJZIGINGEN T.O.V. v3 (13 mei):

- NAT volledig verwijderd van R-EDGE en R-EDGE2 (Yvan-feedback 17 juni: OSPF

werkt niet over NAT; NAT mag alleen op echte internet-edge, niet intern)

- Redistribute uitgebreid in beide richtingen tussen OSPF process 1 en 25

- Route-tags toegevoegd (tag 1 en tag 25) tegen routing loops

- default-information originate VERWIJDERD van R-EDGE2 (alleen R-EDGE mag dit)

- SW-CORE-RACK1 config opgeschoond (oude dubbele/rommelige blokken weg)

- Inter-rack links naar Mehdi (Gig1/0/21) en Adil (Gig1/0/20) toegevoegd

========================================================================

Instructies: plak elke sectie in de CLI van het juiste toestel in Packet Tracer.

Wacht na elke sectie tot je de prompt ziet voordat je de volgende plakt.

========================================================================

1. SW-DIST (Cisco Catalyst 3650) — MASTER (distributie)

========================================================================

enable

configure terminal

hostname SW-DIST

!

! Global settings

ip routing

ipv6 unicast-routing

spanning-tree mode rapid-pvst

spanning-tree vlan 110-199 priority 4096

!

! VLANs

vlan 110

name R1_A_OFFICE

vlan 111

name R1_A_CHECKIN

vlan 112

name R1_A_GATES

vlan 120

name R1_B_OFFICE

vlan 121

name R1_B_CHECKIN

vlan 122

name R1_B_GATES

vlan 130

name R1_VOIP

vlan 131

name R1_IOT

vlan 132

name R1_PRINTERS

vlan 140

name R1_WIFI_STAFF

vlan 141

name R1_WIFI_GUEST

vlan 190

name R1_NET_MGMT

vlan 199

name R1_TRANSIT

exit

!

! SVIs with HSRP + IPv6 + ip helper

interface vlan 110

ip address 10.1.110.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:110::1/64

ipv6 enable

standby 110 ip 10.1.110.1

standby 110 priority 110

standby 110 preempt

no shutdown

interface vlan 111

ip address 10.1.111.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:111::1/64

ipv6 enable

no shutdown

interface vlan 112

ip address 10.1.112.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:112::1/64

ipv6 enable

no shutdown

interface vlan 120

ip address 10.1.120.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:120::1/64

ipv6 enable

standby 120 ip 10.1.120.1

standby 120 priority 110

standby 120 preempt

no shutdown

interface vlan 121

ip address 10.1.121.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:121::1/64

ipv6 enable

no shutdown

interface vlan 122

ip address 10.1.122.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:122::1/64

ipv6 enable

no shutdown

interface vlan 130

ip address 10.1.130.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:130::1/64

ipv6 enable

standby 130 ip 10.1.130.1

standby 130 priority 110

standby 130 preempt

no shutdown

interface vlan 131

ip address 10.1.131.1 255.255.255.0

ip access-group ACL_IOT in

ipv6 address 2001:DB8:1:131::1/64

ipv6 enable

no shutdown

interface vlan 132

ip address 10.1.132.1 255.255.255.0

ipv6 address 2001:DB8:1:132::1/64

ipv6 enable

no shutdown

interface vlan 140

ip address 10.1.140.1 255.255.255.0

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:140::1/64

ipv6 enable

no shutdown

interface vlan 141

ip address 10.1.141.1 255.255.255.0

ip access-group ACL_GUEST_WIFI in

ip helper-address 10.1.190.10

ipv6 address 2001:DB8:1:141::1/64

ipv6 enable

standby 141 ip 10.1.141.1

standby 141 priority 110

standby 141 preempt

no shutdown

interface vlan 190

ip address 10.1.190.1 255.255.255.0

ipv6 address 2001:DB8:1:190::1/64

ipv6 enable

no shutdown

!

! Physical interfaces

interface GigabitEthernet1/0/1

description Trunk naar SW1

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/2

description Trunk naar SW2

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/3

description Trunk naar SW3

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/4

description Uplink naar R-EDGE

no switchport

ip address 192.168.1.2 255.255.255.0

no shutdown

interface GigabitEthernet1/0/5

description Access naar SERVER-RACK1

switchport mode access

switchport access vlan 190

no shutdown

interface GigabitEthernet1/0/6

description Interlink naar SW-DIST2

switchport mode trunk

no shutdown

!

! OSPF process 1 (intern Rack 1)

router ospf 1

router-id 3.3.3.3

network 10.1.0.0 0.0.255.255 area 0

network 192.168.1.0 0.0.0.255 area 0

exit

!

! Default route naar R-EDGE

ip route 0.0.0.0 0.0.0.0 192.168.1.1

!

! ACLs

ip access-list extended ACL_AIRLINE_ISOLATION

deny ip 10.1.110.0 0.0.3.255 10.1.120.0 0.0.3.255

deny ip 10.1.120.0 0.0.3.255 10.1.110.0 0.0.3.255

permit ip any any

!

ip access-list extended ACL_GUEST_WIFI

deny ip 10.1.141.0 0.0.0.255 10.1.0.0 0.0.255.255

permit ip 10.1.141.0 0.0.0.255 any

!

ip access-list extended ACL_IOT

deny ip 10.1.131.0 0.0.0.255 10.1.110.0 0.0.7.255

deny ip 10.1.131.0 0.0.0.255 10.1.120.0 0.0.7.255

permit ip 10.1.131.0 0.0.0.255 10.1.199.0 0.0.0.3

deny ip any any

!

ip access-list extended ACL_MGMT_IN

permit tcp 10.1.190.0 0.0.0.255 any eq 22

permit icmp 10.1.190.0 0.0.0.255 any

deny ip any any

!

! SSH

ip domain-name brussels-airport.local

crypto key generate rsa modulus 2048

ip ssh version 2

username admin privilege 15 secret cisco

line vty 0 15

transport input ssh

login local

exit

!

! NTP

ntp server 10.1.190.10

!

end

write memory

========================================================================

2. SW-DIST2 (Cisco Catalyst 3650) — BACKUP (distributie)

========================================================================

enable

configure terminal

hostname SW-DIST2

!

ip routing

spanning-tree mode rapid-pvst

spanning-tree vlan 110-199 priority 8192

!

vlan 110

vlan 111

vlan 112

vlan 120

vlan 121

vlan 122

vlan 130

vlan 131

vlan 132

vlan 140

vlan 141

vlan 190

exit

!

interface vlan 110

ip address 10.1.110.2 255.255.255.0

standby 110 ip 10.1.110.1

standby 110 priority 100

no shutdown

interface vlan 120

ip address 10.1.120.2 255.255.255.0

standby 120 ip 10.1.120.1

standby 120 priority 100

no shutdown

interface vlan 130

ip address 10.1.130.2 255.255.255.0

standby 130 ip 10.1.130.1

standby 130 priority 100

no shutdown

interface vlan 141

ip address 10.1.141.2 255.255.255.0

standby 141 ip 10.1.141.1

standby 141 priority 100

no shutdown

interface vlan 190

ip address 10.1.190.2 255.255.255.0

no shutdown

!

interface GigabitEthernet1/0/1

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/2

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/3

switchport mode trunk

no shutdown

interface GigabitEthernet1/0/4

no switchport

ip address 10.1.199.1 255.255.255.252

no shutdown

interface GigabitEthernet1/0/6

switchport mode trunk

no shutdown

!

router ospf 1

router-id 5.5.5.5

network 10.1.0.0 0.0.255.255 area 0

network 10.1.199.0 0.0.0.3 area 0

exit

!

ntp server 10.1.190.10

!

end

write memory

========================================================================

3. SW1 (Cisco 2960-24T) — Airline A

========================================================================

enable

configure terminal

hostname SW1

!

spanning-tree mode rapid-pvst

!

vlan 110

name R1_A_OFFICE

vlan 111

name R1_A_CHECKIN

vlan 112

name R1_A_GATES

exit

!

interface range FastEthernet0/1 - 12

switchport mode access

switchport access vlan 110

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/13 - 20

switchport mode access

switchport access vlan 111

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/21 - 23

switchport mode access

switchport access vlan 112

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface FastEthernet0/24

description Trunk naar SW-DIST2

switchport mode trunk

no spanning-tree bpduguard enable

no shutdown

interface GigabitEthernet0/1

description Trunk naar SW-DIST

switchport mode trunk

no shutdown

!

! DHCP snooping

ip dhcp snooping

ip dhcp snooping vlan 110,111,112

no ip dhcp snooping information option

interface GigabitEthernet0/1

ip dhcp snooping trust

!

ntp server 10.1.190.10

!

end

write memory

========================================================================

4. SW2 (Cisco 2960-24T) — Airline B

========================================================================

enable

configure terminal

hostname SW2

!

spanning-tree mode rapid-pvst

!

vlan 120

name R1_B_OFFICE

vlan 121

name R1_B_CHECKIN

vlan 122

name R1_B_GATES

exit

!

interface range FastEthernet0/1 - 12

switchport mode access

switchport access vlan 120

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/13 - 20

switchport mode access

switchport access vlan 121

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/21 - 23

switchport mode access

switchport access vlan 122

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface FastEthernet0/24

description Trunk naar SW-DIST2

switchport mode trunk

no spanning-tree bpduguard enable

no shutdown

interface GigabitEthernet0/1

description Trunk naar SW-DIST

switchport mode trunk

no shutdown

!

ip dhcp snooping

ip dhcp snooping vlan 120,121,122

no ip dhcp snooping information option

interface GigabitEthernet0/1

ip dhcp snooping trust

!

ntp server 10.1.190.10

!

end

write memory

========================================================================

5. SW3 (Cisco 2960-24T) — Gedeelde Zone

========================================================================

enable

configure terminal

hostname SW3

!

spanning-tree mode rapid-pvst

!

vlan 130

name R1_VOIP

vlan 131

name R1_IOT

vlan 132

name R1_PRINTERS

vlan 140

name R1_WIFI_STAFF

vlan 141

name R1_WIFI_GUEST

exit

!

interface range FastEthernet0/1 - 10

switchport mode access

switchport access vlan 130

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/11 - 18

switchport mode access

switchport access vlan 131

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface range FastEthernet0/19 - 21

switchport mode access

switchport access vlan 132

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface FastEthernet0/22

switchport mode access

switchport access vlan 140

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface FastEthernet0/23

switchport mode access

switchport access vlan 141

spanning-tree portfast

spanning-tree bpduguard enable

switchport port-security

switchport port-security maximum 1

switchport port-security mac-address sticky

switchport port-security violation restrict

interface FastEthernet0/24

description Trunk naar SW-DIST2

switchport mode trunk

no spanning-tree bpduguard enable

no shutdown

interface GigabitEthernet0/1

description Trunk naar SW-DIST

switchport mode trunk

no shutdown

!

ip dhcp snooping

ip dhcp snooping vlan 130,131,132,140,141

no ip dhcp snooping information option

interface GigabitEthernet0/1

ip dhcp snooping trust

!

ntp server 10.1.190.10

!

end

write memory

========================================================================

6. R-EDGE (Router-PT) — PRIMARY EDGE

v4: NAT VERWIJDERD + redistribute met tags

========================================================================

enable

configure terminal

hostname R-EDGE

!

interface FastEthernet0/0

description LAN naar SW-DIST

ip address 192.168.1.1 255.255.255.0

no shutdown

interface FastEthernet1/0

description Link naar SW-CORE-RACK1 (inter-rack)

ip address 10.99.0.74 255.255.255.252

ip ospf network point-to-point

no shutdown

interface Serial2/0

description Link naar R-EDGE2

ip address 10.1.199.5 255.255.255.252

no shutdown

!

! --- OSPF process 1 (intern Rack 1) ---

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.1.199.4 0.0.0.3 area 0

redistribute ospf 25 subnets tag 1

exit

!

! --- OSPF process 25 (inter-rack / intraweb) ---

router ospf 25

router-id 1.1.1.1

network 10.99.0.72 0.0.0.3 area 0

redistribute ospf 1 subnets tag 25

exit

!

! Default route naar serial backup pad

ip route 0.0.0.0 0.0.0.0 10.1.199.6

!

end

write memory

! ========================================================================

! VERIFICATIE R-EDGE:

!   show ip nat translations        -> moet LEEG zijn (NAT weg)

!   show ip ospf neighbor            -> 3 FULL: SW-DIST, R-EDGE2, SW-CORE

!   show ip route                    -> 12 VLANs als O, inter-rack als O E2

! ========================================================================

========================================================================

7. R-EDGE2 (Router-PT) — BACKUP EDGE

v4: NAT VERWIJDERD + redistribute met tags + GEEN default-info originate

========================================================================

enable

configure terminal

hostname R-EDGE2

!

interface FastEthernet0/0

description LAN naar SW-DIST2

ip address 10.1.199.2 255.255.255.252

no shutdown

interface FastEthernet1/0

description Link naar SW-CORE-RACK1 (inter-rack)

ip address 10.99.0.78 255.255.255.252

ip ospf network point-to-point

no shutdown

interface Serial2/0

description Link naar R-EDGE

ip address 10.1.199.6 255.255.255.252

clock rate 64000

no shutdown

!

! --- OSPF process 1 (intern Rack 1) ---

! LET OP: GEEN default-information originate hier (alleen op R-EDGE)

router ospf 1

router-id 4.4.4.4

network 10.1.199.0 0.0.0.3 area 0

network 10.1.199.4 0.0.0.3 area 0

redistribute ospf 25 subnets tag 1

exit

!

! --- OSPF process 25 (inter-rack / intraweb) ---

router ospf 25

router-id 4.4.4.4

network 10.99.0.76 0.0.0.3 area 0

redistribute ospf 1 subnets tag 25

exit

!

end

write memory

! ========================================================================

! VERIFICATIE R-EDGE2:

!   show ip nat translations        -> leeg

!   show ip ospf neighbor            -> SW-CORE + R-EDGE FULL

!   show ip route                    -> O E2 routes van andere racks

! ========================================================================

========================================================================

8. SW-CORE-RACK1 (Cisco Catalyst 3650-24PS) — INTER-RACK CORE

v4: opgeschoonde config + Mehdi/Adil links + point-to-point OSPF

========================================================================

enable

configure terminal

hostname SW-CORE-RACK1

no ip domain-lookup

!

! --- L3 routing aanzetten ---

ip routing

vtp mode transparent

!

! --- Link naar Peer0 (richting Aleksander / Rack 2) ---

interface GigabitEthernet1/0/23

description TO-PEER0-ALEKSANDER

no switchport

ip address 10.99.0.2 255.255.255.252

ip ospf network point-to-point

no shutdown

!

! --- Link naar R-EDGE (intern) ---

interface GigabitEthernet1/0/24

description TO-R-EDGE-Fa1/0

no switchport

ip address 10.99.0.73 255.255.255.252

ip ospf network point-to-point

no shutdown

!

! --- Link naar R-EDGE2 (intern) ---

interface GigabitEthernet1/0/22

description TO-R-EDGE2-Fa1/0

no switchport

ip address 10.99.0.77 255.255.255.252

ip ospf network point-to-point

no shutdown

!

! --- Link naar Mehdi (Rack 3) ---

interface GigabitEthernet1/0/21

description TO-METROSW-MEHDI-RACK3

no switchport

ip address 10.99.0.17 255.255.255.252

ip ospf network point-to-point

no shutdown

!

! --- Link naar Adil ---

interface GigabitEthernet1/0/20

description TO-METROSW-ADIL

no switchport

ip address 10.99.0.21 255.255.255.252

ip ospf network point-to-point

no shutdown

!

! --- OSPF process 25 (intraweb tussen racks) ---

router ospf 25

router-id 10.99.0.2

log-adjacency-changes

network 10.99.0.0 0.0.0.3 area 0

network 10.99.0.72 0.0.0.3 area 0

network 10.99.0.76 0.0.0.3 area 0

network 10.99.0.16 0.0.0.3 area 0

network 10.99.0.20 0.0.0.3 area 0

exit

!

end

write memory

! ========================================================================

! VERIFICATIE SW-CORE-RACK1:

!   show ip ospf neighbor   -> R-EDGE + R-EDGE2 FULL (+ Aleksander/Mehdi/Adil

!                              zodra hun multiuser-sessie verbonden is)

!   show ip route           -> alle 12 VLANs als O E2 (via redistribute op edge)

!   show ip interface brief -> Gig1/0/22 en Gig1/0/24 up/up

! ========================================================================

========================================================================

INTER-RACK IP-PLAN (referentie, volgens master IntraWeb-Ports)

========================================================================

SW-CORE-RACK1 naar Aleksander (Peer0):  10.99.0.2/30   (Aleksander = 10.99.0.1)

SW-CORE-RACK1 naar Mehdi (Rack 3):      10.99.0.17/30  (Mehdi = 10.99.0.18)

SW-CORE-RACK1 naar Adil:                10.99.0.21/30  (Adil = 10.99.0.22)

SW-CORE-RACK1 naar R-EDGE (intern):     10.99.0.73/30  (R-EDGE = 10.99.0.74)

SW-CORE-RACK1 naar R-EDGE2 (intern):    10.99.0.77/30  (R-EDGE2 = 10.99.0.78)

OSPF-architectuur:

Process 1   = intern Rack 1 (SW-DIST, SW-DIST2, R-EDGE, R-EDGE2)

Process 25  = inter-rack (SW-CORE-RACK1, R-EDGE, R-EDGE2, andere racks)

Redistribute in beide richtingen met route-tags (tag 1 en tag 25) tegen loops

default-information originate alleen op R-EDGE (niet op R-EDGE2)